# Motor Subtraction ERP Pipeline code review & Correction

Project: EEG Motor Subtraction ERP Pipeline

Subject package: sub-01

Date: 2026-03-10

## Summary

This document accompanies the submitted sub-01 results package and summarizes:

- The corrected scripts delivered.

- What was wrong and what was changed.

- The example outputs generated for one subject.

- Additional pipeline review findings.

The pipeline has been re-run and validated on sub-01, and the output artifacts were generated successfully.

## Deliverable 1: Corrected Scripts

The following scripts were updated as part of the fix set:

- eeglab_ERP_analysis/subtract_motor_and_compute_difference.m

- eeglab_ERP_analysis/PrepareData_7_ComputeERPs.m

- eeglab_ERP_analysis/plot_action_noaction_comparison.m

- eeglab_epochs/compare_N1_P2_amplitudes.m

- eeglab_ERP_analysis/plot_scalp_topographies.m

- eeglab_epochs/PrepareData_6_ExtractConditions.m

Primary correction target:

- subtract_motor_and_compute_difference.m was corrected and hardened to ensure reliable time alignment, subtraction behavior, and deterministic difference-wave computation.

## Deliverable 2: Detailed Problem Analysis and Exact Changes

### 2.1 Root-cause summary

The primary issue was not a single syntax/runtime failure. The core subtraction flow existed, but downstream integration inconsistencies caused scientifically unreliable condition comparisons.

Most important root causes:

- Some comparison scripts mixed processed Action data with raw NoAction data.

- One script loaded motor-subtracted Action from the wrong folder.

- Wrapper orchestration allowed metrics/plots to be generated before final harmonized data selection.

- One plotting script expected field names that did not match producer outputs.

- Small Action/NoAction endpoint differences (1 sample) could produce fragile subtraction branches without explicit grid harmonization.

### 2.2 Exact edits Done

This section lists concrete file-level edits and their purpose.

#### A) eeglab_ERP_analysis/subtract_motor_and_compute_difference.m

Problem addressed:

- Subtraction behavior and cross-condition comparison required deterministic, trial-preserving logic and robust handling of tiny time-grid mismatches.

Exact edits:

- Implemented trial-level subtraction explicitly:

  - template_3d = repmat(motor_template, [1 1 EEG_kp_bc.trials]);

  - EEG_subtracted.data = EEG_kp_bc.data - template_3d;

- Preserved a collapsed signal from cleaned trial data for downstream grand-average visualization:

  - EEG_subtracted.collapsed_erp_clean = mean(EEG_subtracted.data, 3);

- Applied final tone-locked baseline correction to both conditions in one location using the same target window ([-200, 0] ms) with per-dataset clamping.

- Saved processed outputs to ERP output path:

  - Action: *_main_action_motor_subtracted.set/.fdt

  - NoAction: *_main_bc.set/.fdt

- Added deterministic Action/NoAction time-grid harmonization before subtraction/difference wave:

  - If time vectors differ, NoAction is interpolated to Action grid using interp1(..., 'linear', 'extrap').

- Computed per-tone and collapsed difference waves from harmonized data and saved:

  - sub-01_difference_wave_motor_subtracted.mat

Why this was done.

- Preserves trial structure.

- Prevents edge-case subtraction instability when one condition is shorter by one sample.

- Ensures Action and NoAction are compared on a common grid.

#### B) eeglab_ERP_analysis/PrepareData_7_ComputeERPs.m

Problem addressed:

- Orchestration risk of mismatched processing states in final metrics/plots.

Exact edits:

- Runs motor subtraction early in the pipeline flow when Action main + baseline are available.

- Reloads motor-subtracted Action main from ERP output path after subtraction:

  - <out_path>/<Sub>_ses-02_task-action_eeg_main_action_motor_subtracted.set

- Prefers processed NoAction main from ERP output path when available:

  - <out_path>/<Sub>_ses-01_task-no-action_eeg_main_bc.set

- Continues with fallback behavior if processed files are unavailable, but now defaults to matched processed datasets.

- Keeps final result export consolidated in ERP output folder (.mat, .csv, .xlsx, figures).

Why this was done:

- Reduces the risk of plotting/quantifying Action from processed data against raw NoAction.

#### C) eeglab_ERP_analysis/plot_action_noaction_comparison.m

Problem addressed:

- Comparison plot input inconsistency.

Exact edits:

- NoAction main file selection now prefers processed baseline-corrected main in ERP folder:

  - first: <out_path>..._main_bc.set

  - fallback: <epo_path>..._main.set

- Action file selection now prefers motor-subtracted Action main in ERP folder:

  - first: <out_path>..._main_action_motor_subtracted.set

  - fallback: raw <epo_path>..._main_action.set

- Retained collapsed and per-tone plotting logic, but now driven by corrected file-priority rules.

Note:

- Ensures visual comparison figures are based on matched processing state wherever available.

#### D) eeglab_epochs/compare_N1_P2_amplitudes.m

Problem addressed:

- Wrong path for Action motor-subtracted load and inconsistent NoAction source.

Exact edits:

- Changed Action main load priority to ERP output path for motor-subtracted data:

  - first: <out_path>..._main_action_motor_subtracted.set

  - fallback: raw <epo_path>..._main_action.set

- Changed NoAction main load priority to ERP output path baseline-corrected data:

  - first: <out_path>..._main_bc.set

  - fallback: raw <epo_path>..._main.set

- Updated header description to match implemented behavior:

  - from inferential/paired-test wording to descriptive within-subject summary wording.

Why this matters:

- Prevents missing or wrong-input Action main files in downstream amplitude summary.

- Improves parity between compared conditions.

#### E) eeglab_ERP_analysis/plot_scalp_topographies.m

Problem addressed:

- Producer/consumer schema mismatch.

Edits:

- Updated expected latency fields to match compute_N1_P2.m outputs:

  - results.Central.N1_peak_lat

  - results.Central.P2_peak_lat

- Removes runtime mismatch and ensures topographies are sampled at intended peak latencies.

#### F) eeglab_epochs/PrepareData_6_ExtractConditions.m

Problem addressed:

- Runtime messaging did not clearly reflect configured-vs-default epoch windows.

Exact edits:

- Replaced hardcoded/ambiguous window status text with cfg-aware logging:

  - if cfg.epoch_tmin/epoch_tmax present, print those values.

  - otherwise print that extraction defaults are used.

- Improves reproducibility traceability in logs and reduces confusion during reruns.

### 2.3 Before/After behavioral summary

Before fixes:

- High risk of processed-vs-raw mixing in Action vs NoAction outputs.

- Potential wrong-path load for motor-subtracted Action in one analysis script.

- Field mismatch risk in topography plotting.

- Time-axis mismatch handling could be fragile in edge cases.

After fixes:

- Processed file preference standardized across comparison pathways.

- Action motor-subtracted and NoAction baseline-corrected outputs are consistently consumed when present.

- Topography field mapping is aligned with metric producer schema.

- Difference-wave computation is stabilized with explicit time-grid harmonization.

Net result:

- sub-01 end-to-end outputs are internally consistent with the corrected processing logic and are reproducibly generated under the expected output structure.

## Deliverable 3: Example Outputs From One Subject (sub-01)

The following expected outputs were generated under eeglab_ERPs/sub-01/:

### Core data outputs

- eeglab_ERPs/sub-01/sub-01_ses-02_task-action_eeg_main_action_motor_subtracted.set

- eeglab_ERPs/sub-01/sub-01_ses-02_task-action_eeg_main_action_motor_subtracted.fdt

- eeglab_ERPs/sub-01/sub-01_ses-01_task-no-action_eeg_main_bc.set

- eeglab_ERPs/sub-01/sub-01_ses-01_task-no-action_eeg_main_bc.fdt

- eeglab_ERPs/sub-01/sub-01_difference_wave_motor_subtracted.mat

- eeglab_ERPs/sub-01/sub-01_ERP_results.csv

- eeglab_ERPs/sub-01/sub-01_ERP_results.xlsx

### Required example figures

- Motor template:

  - eeglab_ERPs/sub-01/figures/sub-01_motor_template.png

- Motor-subtracted Action ERP:

  - eeglab_ERPs/sub-01/figures/sub-01_action_main_motor_subtracted.png

- No-Action ERP:

  - eeglab_ERPs/sub-01/figures/sub-01_NoAction_Main_grand_average.png

- Difference wave:

  - eeglab_ERPs/sub-01/figures/sub-01_diff_wave_motor_subtracted_per_tone.png

  - eeglab_ERPs/sub-01/figures/sub-01_diff_wave_motor_subtracted_all.png

Additional comparison views included:

- eeglab_ERPs/sub-01/figures/sub-01_Action_vs_NoAction_Collapsed.png

- eeglab_ERPs/sub-01/figures/sub-01_Action_vs_NoAction_PerTone.png